# DIGITAL CLOCK

## LED 7-Segment Display

### Using Arduino Nano | Multiplexing | Manual Time Control

## 1. Project Overview

This project demonstrates a fully functional digital clock built using discrete red LEDs arranged as custom 7-segment displays on a prototype PCB (dot PCB). Each digit of the clock is formed by illuminating specific LED segments, replicating the classic 7-segment numeral pattern.

An Arduino Nano controls all display segments using multiplexing, rapidly refreshing each digit so the human eye perceives continuous illumination.

The clock displays hours:minutes in HH:MM format and includes push-button controls for manual time setting.

The complete display was handmade using 92 red LEDs:

- 88 LEDs for the 4-digit display

- 4 LEDs for the dual blinking colon indicators

## 2. System Components

| Component | Specification | Qty | Purpose |
| --- | --- | --- | --- |
| Arduino Nano | ATmega328P, 5V, 16MHz | 1 | Main controller & timekeeping |
| Red LEDs | 5mm LEDs | 92 | Display segments & colon indicators |
| Prototype PCB / Dot PCB | Copper board | 1 | Mounting & soldering |
| Push Buttons | Momentary SPST | 3 | Mode / Time control |
| Wires |  | As required | Connections |
| USB Power Supply | 5V | 1 | Power source |

## 3. 7-Segment Display Theory

A standard 7-segment display uses seven LED segments labelled A through G. By illuminating different combinations of these segments, decimal numbers from 0 to 9 can be displayed.

### Segment Layout

- Segment A → Top horizontal

- Segment B → Top-right vertical

- Segment C → Bottom-right vertical

- Segment D → Bottom horizontal

- Segment E → Bottom-left vertical

- Segment F → Top-left vertical

- Segment G → Middle horizontal

The display uses a common-anode multiplexed configuration.

## 4. How Multiplexing Works

Driving four digits independently would normally require a large number of Arduino pins. Multiplexing reduces pin usage by sharing segment lines across all digits and enabling only one digit at a time.

The Arduino rapidly cycles through:

- Enable one digit

- Send segment data

- Disable digit

- Move to next digit

This process repeats continuously at high speed, creating the illusion that all digits are ON simultaneously.

## 5. Circuit & Pin Connections

### Digit Pins

| Arduino Pin | Connected To | Function |
| --- | --- | --- |
| D2 | Digit 1 | Hours Tens |
| D3 | Digit 2 | Hours Units |
| D4 | Digit 3 | Minutes Tens |
| D5 | Digit 4 | Minutes Units |

### Segment Pins

| Arduino Pin | Segment |
| --- | --- |
| D6 | Segment A |
| D7 | Segment B |
| D8 | Segment C |
| D9 | Segment D |
| D10 | Segment E |
| D11 | Segment F |
| D12 | Segment G |

### Colon Indicator

| Arduino Pin | Function |
| --- | --- |
| D13 | Colon LEDs |

### Push Buttons

| Arduino Pin | Function |
| --- | --- |
| A3 | MODE |
| A4 | UP |
| A5 | DOWN |

## 6. Working Principle

The Arduino Nano continuously refreshes the multiplexed display while maintaining software based timekeeping using the millis() timer function.

The clock operates in three modes:

- Normal Clock Mode

- Hour Edit Mode

- Minute Edit Mode

The MODE button cycles through these states:

- Normal → Hours → Minutes → Normal

The UP and DOWN buttons are used to adjust the selected time value.

The blinking colon LEDs act as a visual seconds indicator.

## 7. Button Functions

| Button | Function |
| --- | --- |
| MODE | Enter setting mode / switch between hours and minutes |
| UP | Increase selected value |
| DOWN | Decrease selected value |

## 8. Software Logic

The program uses:

- Multiplexing for display control

- millis() based timing system

- Internal pull-up resistors for button inputs

- State based mode switching

- Manual time editing system

While in edit mode, the clock pauses time counting until the user exits back to normal mode.

## 9. Technical Specifications

| Parameter | Value |
| --- | --- |
| Microcontroller | Arduino Nano |
| Display Type | Custom LED 7-Segment |
| Display Format | HH:MM |
| LED Count | 92 Red LEDs |
| Display Configuration | Common Anode |
| Supply Voltage | 5V DC |
| Timekeeping Method | Software millis() |
| User Interface | Push Buttons |
| Board Type | Prototype PCB / Dot PCB |

## 10. Advantages

- Large custom display design

- Low cost implementation

- Hands-on multiplexing experience

- Fully repairable LED structure

- Expandable architecture

- Educational embedded systems project

- Customizable display size and color

## 11. Applications

- DIY digital clocks

- Embedded systems learning

- Electronics project demonstrations

- Multiplexing experiments

- Educational Arduino projects

## 12. Future Improvements

- Add RTC module for accurate timekeeping

- Add alarm functionality

- Add brightness control using PWM

- Add battery backup

- Add WiFi time synchronization

- Design custom PCB

- Add temperature display mode

## 13. Final Conclusion

This project demonstrates how a fully functional digital clock can be built using discrete LEDs, multiplexing techniques, and embedded programming concepts.

By combining custom hardware construction with software controlled display management, the project provides practical understanding of:

- LED segment encoding

- Multiplexing

- Timekeeping

- Push button interfacing

- Real-time embedded systems

The handmade prototype PCB construction and large custom LED display make the project both visually appealing and educationally valuable.